Camera Validation Report
Full-spectrum spectrometer comparison with filtered camera measurements

Main conclusion The camera produces structured, wavelength-dependent data and tracks the spectrometer well in several matched sample series. The current dataset supports a credible proof of concept, especially with per-sample normalized fingerprints. It is not yet a calibration-grade proof across every sample because exposure/reference control appears inconsistent.

| Measurement | Value |
| --- | --- |
| Paired camera-spectrum measurements | 28 |
| Per-sample fingerprint comparisons | 5 |
| Median camera/spectrometer fingerprint correlation | 0.603 |
| Full-spectrum classifier accuracy | 41.7% |
| PCA variance explained by PC1 + PC2 | 61.0% |

# Executive Summary

The new TXT files contain real spectrometer data: each spectrum has 2048 wavelength-intensity points across the full measured range.

The analysis intentionally uses the complete spectrum for full-spectrum area, normalized curve shape, centroid, PCA, and similarity calculations.

The strongest camera-vs-spectrometer evidence is the per-sample fingerprint comparison: several sample series show moderate to strong tracking between camera response and spectrometer full-spectrum response.

The median per-sample normalized camera/spectrometer correlation is 0.603.

The global raw correlation is weak, which is expected if camera exposure, white balance, sample placement, or reference conditions changed between captures.

A naive full-spectrum healthy/unhealthy classifier is weak in this dataset, so the report does not claim disease classification is solved. The defensible claim is that the camera captures useful wavelength-dependent signal that can be aligned to spectrometer measurements under controlled conditions.

## Most Useful Evidence For Your App

| Sample | Condition | Filters | Camera vs spectrum r | Interpretation |
| --- | --- | --- | --- | --- |
| Set frunze 1/frunza nesanatoasa | unhealthy | 5 | 0.772158 | strong match |
| Set frunze 1/frunza verde | healthy | 6 | 0.62609 | moderate match |
| Set frunze 2/nesanatoasa | unhealthy | 5 | -0.139189 | weak or inconsistent |
| Set frunze 2/sanatoasa | healthy | 6 | 0.603096 | moderate match |
| frunza sanatoasa dar galbena | healthy_yellow | 6 | -0.540308 | weak or inconsistent |

# Method

The analysis parsed all available spectrometer TXT files and camera JPG files from the real data folder. For every matched camera/spectrum pair, the matching key was sample folder plus filter label. The spectra were smoothed with a Savitzky-Golay filter, but no wavelength range was removed; the full measured curve was kept for full-spectrum features.

| Feature | Meaning |
| --- | --- |
| Full-spectrum area | Integral of the smoothed, dark-corrected positive signal over the entire measured wavelength range. |
| Normalized spectrum | Z-normalized smoothed curve using all 2048 points. |
| Camera brightness | Mean leaf-pixel brightness from the segmented camera image. |
| Fingerprint correlation | Correlation across filters between camera response and spectrometer full-spectrum area within the same sample. |
| PCA | Dimensionality reduction of the complete normalized spectrum, not selected bands. |

# Findings

## 1. The camera signal is structured, not random

The filtered camera images produce repeatable changes across filter labels. This is visible in the fingerprint plots and camera feature heatmap. The signal is not merely noise; it changes with filter, sample, and condition.

## 2. The spectrometer confirms strong wavelength-dependent structure

The spectrometer curves show strong filter-dependent peaks and broad full-spectrum differences. This makes them suitable as a reference instrument for validating the camera response.

## 3. Per-sample comparison is the right proof strategy

Raw global correlation is weak because the dataset combines different leaves, conditions, references, and likely camera exposure states. When each sample is normalized internally and compared as a response fingerprint, several samples show useful agreement.

Set frunze 1/frunza nesanatoasa: r=0.772, R^2=0.596.

Set frunze 1/frunza verde: r=0.626, R^2=0.392.

Set frunze 2/sanatoasa: r=0.603, R^2=0.364.

## 4. The present dataset does not prove universal calibration yet

Some sample series are weak or inverted, and the full-spectrum classifier does not separate healthy/unhealthy reliably (accuracy 41.7%). This does not mean the camera is useless; it means the proof should be framed as a working, wavelength-sensitive camera prototype, not a finished calibrated instrument.

# Core Figures

Figure 1. Mean full-spectrum curves by condition.

Figure 2. Full-spectrum difference curve: healthy minus unhealthy.

Figure 3. PCA of complete normalized spectra.

Figure 4. Within-sample normalized camera response versus spectrometer full-spectrum response.

Figure 5. Per-sample correlation between camera and spectrometer fingerprints.

Figure 6. Camera and spectrometer response fingerprints by sample.

Figure 7. Spectrometer full-spectrum area by condition and filter.

Figure 8. Full-spectrum similarity heatmap.

# How To Use This In The App

Use the camera as a multispectral response sensor by comparing response fingerprints rather than isolated raw brightness values. In the app, every sample should produce both a camera fingerprint and, when available, a spectrometer fingerprint.

Segment the leaf in each filtered camera image.

Compute camera brightness, saturation, GCC, and other image features.

Normalize those features within the sample across filters.

For spectrometer validation, compute full-spectrum area and normalized full-spectrum shape using all wavelengths.

Report camera/spectrometer fingerprint correlation as a confidence score.

Flag low-correlation samples for retake or calibration review.

Best proof-of-concept claim The camera captures wavelength-dependent leaf response patterns that can be compared to a spectrometer. Under controlled sample series, the camera fingerprint moderately to strongly tracks the spectrometer full-spectrum fingerprint.

# Limitations And Next Measurements

The JPG files do not expose useful EXIF exposure metadata, so changing exposure/white balance cannot be corrected after the fact.

White-reference correction helps some samples but hurts others, suggesting the reference setup was not fully matched to every sample.

The camera and spectrometer likely do not observe exactly the same spatial region of the leaf.

Some classes are confounded with filter and environment. A naive classifier should not be used as the headline proof.

## Recommended retake protocol

Lock camera exposure, ISO, focus, and white balance.

Capture white and dark references for every filter in the same session.

Use the same leaf ROI for camera and spectrometer measurements.

Take at least three repeats per condition/filter.

Keep filenames machine-readable: sample_condition_filter_repeat.

# Graph Appendix

All candidate graphs were generated as separate PNG files in the graphs folder. Use this list to choose which ones to include in a final presentation or app documentation.

| Graph file | Title |
| --- | --- |
| 01_full_spectrum_raw_all.png | Raw full spectra, all samples |
| 02_full_spectrum_normalized_by_condition.png | Smoothed normalized full spectra by condition |
| 03_condition_mean_full_spectrum.png | Condition mean full-spectrum curves |
| 04_healthy_minus_unhealthy_difference.png | Healthy minus unhealthy full-spectrum difference |
| 05_full_spectrum_pca.png | Full-spectrum PCA score plot |
| 06_camera_vs_spectrometer_full_area.png | Camera brightness vs full-spectrum area |
| 07_camera_vs_spectrometer_normalized.png | Within-sample normalized camera vs spectrometer response |
| 08_camera_reflectance_proxy_vs_spectrometer.png | White-referenced camera proxy vs spectrometer response |
| 09_per_sample_correlation_bars.png | Per-sample camera/spectrometer fingerprint correlations |
| 10_fingerprint_overlays_by_sample.png | Camera and spectrometer response fingerprints by sample |
| 11_camera_feature_heatmap.png | Camera feature heatmap |
| 12_spectrometer_feature_heatmap.png | Spectrometer feature heatmap |
| 13_reference_spectra.png | White/dark/source reference spectra |
| 14_peak_and_centroid_by_filter.png | Peak and centroid by filter |
| 15_classifier_confusion_matrix.png | Full-spectrum nearest-centroid classifier confusion matrix |
| 16_condition_camera_separation.png | Camera feature separation by condition |
| 17_spectrum_area_by_condition_filter.png | Full-spectrum area by condition and filter |
| 18_full_spectrum_similarity_heatmap.png | Full-spectrum similarity heatmap |
| 19_camera_contact_sheet.png | Camera contact sheet |